CM110 8.5 LABVIEW DEMO

QUICK TUTORIAL

Version: 8.5

Name: Cm8C.exe

Getting Started:

- Connect the RS232 cable that was provided to the CM110 back plate and to an identifiable comport on the back of your computer.

- Connect the power cable to the CM110 and power-up.

- After reset, start the Labview program by clicking on the Cm8C.exe that is located on the software CD we provided.

After selection click “SET and Exit”		Check values for communication validity.

Common Functions:

Set Unit of Measure

Enter Wavelength in proper unit of measure (Run then Exit)

Note# To check calibration:

With Tungsten light source attached, enter 0 in wavelength and see if the optimized tungsten light is coming through the exit slit, then proceed with your wavelength test.

Select grating and click “Run and Exit after reset”

Uploading Factory Values when necessary:

Your factory values are located within this CD (23029nt.nov)

Note# Exit program and Power Reset CM110